1. Institutional information

Name of institution (Insert name of your museum here)

Date of completion (Insert date here)

Date of next update of plan 1 year from above date

List of locations where this plan is on file (on and off premises)

(Insert name of plan holder and location here) (on premises)

(Insert name of plan holder and location here) (off premises)

Staff members with a copy of this emergency plan

(Insert name of plan holder and job title here)

Staff members familiar with the contents of the emergency plan

(Insert name and job title here)

7Location of salvage areas

(Insert details here):on site

(Insert details here):off site

2. Essential services at (insert name of your museum here)

2.1 Contact details

Electricity

• Supplier:

• Day tel:

• Out of hours tel:

• Customer Ref:

Gas

• Supplier

• Day tel:

• Out of hours tel:

• Customer Ref:

If you smell gas:

Call Transco (24 hour gas emergency service): 0800 111 999

Undertake the following procedures:

• Don’t smoke or strike matches

• Don’t turn electrical switches on or off

• Do put out naked flames

• Do open windows and doors

• Do keep people away from the affected area

• Do turn the meter off at the control valve

Water

• Supplier:

• Day tel:

• Out of hours tel:

• Customer Ref:

Insurance

• Insurance contact name:

• Day Tel:

• Mobile:

• Policy No:

Police

Local station:

• Day tel:

• Out of hours tel:

In an emergency dial 999

Fire Station

Local station:

• Day tel:

• Out of hours tel:

In an emergency dial 999

2.2 Service control points located in your museum:

(The following is only a guide and can be altered as appropriate)

Stop cocks (water)

Location: (insert details here)

Electricity mains switch

Location: (insert details here)

Fire alarm panel

Location: (insert details here)

Security alarm panel

Location: (insert details here)

Manhole

Location: (insert details here)

Fusebox / Circuit stop

Location: (insert details here)

10

2.3 Site plan

Insert floor plan of the museum (all public and non-public areas) here.

Mark the service control points. Include the location of all break glass fire points, fire extinguishers and fire blankets. Colour code the information about the fire

extinguishers to show what type of extinguisher it is (water, carbon dioxide,

etc). Several additional copies of this plan can be produced, laminated and

kept in the emergency plan. In the event of an emergency, the Fire Service

will be able to identify where fire-fighting equipment is situated. You could also

include any risk information that the fire brigade would need to know.

11

3. Emergency procedures at (insert name of your museum here)

3.1 Fire alarm and evacuation procedures

Fire alarm details

For this building:

• When the fire alarm sounds, it is in the form of (insert details specific to

your fire alarm here, e.g. a continuous ringing bell)

• The fire alarm is tested routinely every (insert name of day here) at

(insert time here)

Basic procedures

On discovering a fire:

• Raise the alarm by using the Break Glass call points situated around

the building. This will inform other people in the building. (Delete the

following in accordance to the situation in your institution:

• Do not attempt to use fire-fighting equipment (e.g. extinguishers),

unless you have been specifically trained to do so and you feel

confident that you can do so. Do not take personal risks.

• Do not attempt to rescue any objects whilst leaving the building, either

from display or storage areas. Salvage should only commence once

the Fire Brigade have the situation under control.

• Make your way to a Fire Exit, closing doors behind you if possible. Go

to the designated assembly point outside (insert the name of the fire

assembly point here).

On hearing the fire alarm:

• Make your way to a Fire Exit, closing doors behind you if possible. Go

to the designated assembly point outside (insert the name of the fire

assembly point here).

• Do not attempt to rescue any objects whilst leaving the building, either

from display or storage areas. Salvage should only commence once

the person in charge has undertaken a risk assessment.

Evacuation procedures

(Insert your evacuation procedure information here).

12

3.2 Key control procedures

The following table shows the sets of keys within the museum (insert more

rows if needed)

Room name / no.

Key name / no.: (Insert details here)

Designated key holder’s name: (Insert details here)

Key holder’s home tel: (Insert details here)

Work tel: (Insert details here)

Mobile: (Insert details here)

Alternative key holder’s name: (Insert details here)

Key holder’s home tel: (Insert details here)

Work tel: (Insert details here)

Mobile: (Insert details here)

Room name / no.

(Insert as many sections as required)

Vehicle name / no.

(Think about any local authority, other museum, friends or neighbour’s

vehicles that you may be able to arrange to use in the event of an emergency)

Key name / no.: (Insert details here)

Designated key holder’s name: (Insert details here)

(Continue to insert the same information as above)

13

4. Health & safety procedure during an

emergency at (insert name of your museum here)

The person in charge must undertake dynamic risk assessments as required.

4.1 First aid

Staff members who are currently trained in first aid:

(Insert name here)

Job title: (Insert details here)

Museum department / section: (Insert details here)

Work tel: (Insert details here)

Mobile tel: (Insert details here)

First aider’s home tel: (Insert details here)

Level of first aid training: (Insert details here)

Last training received: (Insert details here)

Training renewal date due: (Insert details here)

(Insert as many sections as appropriate)

4.2 First aid kits

First aid kits are situated in the following areas of the museum:

• (insert room name and number here)

• (Insert as many sections as appropriate)

In the case of an emergency, or as advised by a first aider

contact the emergency services: dial 999

4.3 Health and Safety during Salvage

Electricity

If you need to switch off your electricity supply and there is water present, get

a qualified person to do it. Do not touch sources of electricity when standing

in water. Backup lighting may need to be in place before salvage starts. The

person in charge must assess the situation before you start salvage work

14

Flood water

Flood water can contain sewage, chemicals and animal waste:

• Wear waterproof outerwear, including gloves and wellington boots.

• A face mask and goggles are advised.

• If mould is present then the face mask must be of a grade that filters

mould spores.

• Do not let flood water come into contact with open wounds or grazes: if

it does, obtain an anti-tetanus injection as soon as possible if you don’t

have an up to date one, or are unsure.

• Always scrub and disinfect hands before eating.

• Do not work in deep or fast flowing water.

• Be aware of concealed hazards such as broken glass or slippery silt

underfoot.

• Be aware of the effects of cold water over time e.g. reduced dexterity

and mobility; regular breaks may be needed.

Fire

Fire can create dangerous environments by releasing hazardous chemicals. If

you have identified these in advance you may be able to salvage with the

correct procedure and equipment.

5. Emergency response team

5.1 Contact details

Overall Co-ordinator

Name: (Insert name here)

Work tel: (Insert number here)

Home tel: (Insert number here)

Mobile: (Insert number here)

Specific Responsibility in Case of Emergency:

• Contacts insurers

• Stays in one place and is available to make decisions

• Undertakes continuous risk assessments

• Acts as, or appoints media contact

• Supports team leaders

• Records events or delegates

Deputy

Name: (Insert name here)

Work tel: (Insert number here)

Home tel: (Insert number here)

Mobile: (Insert number here)

Emergency Services Liaison

Name: (Insert name here)

Work tel: (Insert number here)

Home tel: (Insert number here)

Mobile: (Insert number here)

Specific Responsibility in Case of Emergency:

• Liaison with emergency services, Co-ordinator and Salvage Coordinator

• Responsible for security of site

• Responsible for control of access to the site

16

Deputy

Name: (Insert name here)

Work tel: (Insert number here)

Home tel: (Insert number here)

Mobile: (Insert number here)

(Continue to add details of emergency response team members here.)

17

6. Volunteer Response Team

6.1 Contact Details

Volunteer 1.

Name: (Insert name here)

Work tel: (Insert number here)

Home tel: (Insert number here)

Mobile: (Insert number here)

Any specialist knowledge/ skills/ contacts that may be used in the case of an

emergency?

Volunteer 2.

Name: (Insert name here)

Work tel: (Insert number here)

Home tel: (Insert number here)

Mobile: (Insert number here)

Any specialist knowledge/ skills/ contacts that may be used in the case of an

emergency?

(Continue to add details of volunteer emergency response team members

here)

18

7. (Insert name of room or area here) Emergency plan

7.1 Fire

Basic procedures

On discovering a fire:

• Raise the alarm by using the Break Glass call points situated around

the building. This will inform other people in the building.

• Do not attempt to use fire-fighting equipment (e.g. extinguishers),

unless you have been specifically trained to do so and you feel

confident that you can do so. Do not take personal risks.

• Make your way to a Fire Exit, closing doors behind you if possible. Go

to the designated assembly point (insert the name of the fire assembly

point here).

• Do not attempt to rescue any objects whilst leaving the building,

either from display or storage areas. Salvage should only commence

once instructed.

• The Fire Brigade will investigate the situation and inform the

Emergency Services Liaison when the building is safe to enter, and by

whom.

On hearing the fire alarm:

• Make your way to a Fire Exit, closing doors behind you if possible. Go

to the designated assembly point outside (insert the name of the fire

assembly point here).

• Do not attempt to rescue any objects whilst leaving the building,

either from display or storage areas. Salvage should only commence

once instructed.

• The Fire Brigade will investigate the situation and inform the

Emergency Services Liaison when the building is safe to enter, and by

whom.

Risks to objects

In the event of a fire, objects are at risk from the following:

• Being completely destroyed

• Being partially burnt

• Suffering from soot deposits

• Being water damaged from the fire fighting

Water damage resulting from fire fighting:

• Assess the objects that are most at risk / badly damaged. Take into

consideration any that are on the Priority for salvage floor plan

19

Where are you going to move objects to? (i.e. treatment areas to assess

their condition):

Either of the following:

• (insert name of room or area) (onto floor or table, both to be padded

with blankets or foam, then covered in tissue if objects are dry OR

blankets or foam and polythene if objects are damp/wet).

• (insert name of alternative room or area) (onto floor or table, both to be

padded with blankets or foam, then covered in tissue if objects are dry

OR blankets or foam and polythene if objects are damp/wet).

(Think about any local authority, other museum, friends or neighbour’s

buildings that you may be able to arrange to use in the event of a major

emergency where you need to move collections off site)

IMPORTANT: dry objects must be kept away from damp/wet objects. This

needs to be assessed as objects are taken to treatment areas. Ideally there

should be two areas: dry treatment area and damp/wet treatment area.

• Prepare treatment areas before objects are moved.

What do you need to move the objects?:

• Security screw drivers/ display case keys. Cases should remain

closed until you are ready to remove the objects.

• Latex/ nitrile gloves should be worn at all times when handling

objects.

• Scalpel/small scissors (to remove objects that are tied onto mounts).

Do all objects need to be removed from mounts or can they be moved

as they are?

• Trolleys for moving boxes/crates.

• Storage boxes/ crates for moving smaller objects. If there are not

enough, objects will have to be unpacked at the treatment area and the

boxes re-used.

(Adapt this section for the specific room or area)

IMPORTANT: Wet/ damp organic materials (e.g. wood, leather, paper, textile)

are at risk from mould growth. Objects should be air-dried within 48 hours to

prevent mould growth. If there is a large volume of materials that cannot be

dried within this time, undertake freezing (not suitable for all materials).

Contact a conservator (as recommended on your contacts list) who will

advise on this.

Follow the salvage notes for materials in this emergency pack, to see how

materials should be treated after an emergency. Some materials require slow

drying, as quick drying could cause serious damage.

20

Smoke damage:

• Ensure that a Conservator removes any soot as soon as possible as it

is acidic.

• loose soot: remove carefully with a conservation vacuum (do not use

a normal domestic vacuum cleaner), used at a lower suction power

(fitted with gauze on the nozzle).

• ingrained soot: attempt to use a soot sponge (under the supervision

of a conservator).

• If the object is wet: rinse with clear water.

• If objects are dry: send to the dry object treatment area. If damp / wet,

send to the relevant treatment area.

• A conservator will advise on further treatments.

Documentation during salvage:

• Keeping track of objects as they move is very important.

• Use the Recovery Sheet to record basic information about each

object.

• Labelling objects will help identify them.

• Digital photos can be taken to aid identification.

• Keep any detached labels with the objects (if possible). Paper labels

can be tied to objects whilst they are in the treatment areas.

21

Salvage notes

Air-drying:

• Can be undertaken with objects laid out over an area.

• Bread crates (stackable) can be used to lay objects in, as they allow

air to circulate.

• A wind-tunnel can be created to aid drying but is not for use with

objects that require slow air-drying. For example, polythene sheets

can be draped over tables and the objects laid out beneath the table. A

fan can be placed underneath the table at one end but the air must not

directly blow onto any objects. Objects that require quicker drying can

be placed nearer the fan.

Dehumidifiers:

• Can be used in rooms to help dry out rooms that have been damaged

by flooding (or are water damaged after a fire).

• If used, the doors and windows of that room must be closed.

• The dehumidifier(s) must be plumbed in/or be emptied regularly but

they must not collect the water in open containers, as the moisture will

just be recycled back into the room.

• Monitor the rate of drying, as drying too quickly can cause damage.

DO NOT ADD HEAT DUE TO INCREASED RISK OF MOULD GROWTH

SEE NEXT SECTION FOR: SALVAGE NOTES ON SPECIFIC MATERIALS

22

Salvage notes

• If objects are dry: keep them away from any damp/wet objects.

• If objects are damp/ wet: contact your Museums Development Officer

who can provide you with the details of a Conservator, who will be able

to advise you (if he/she has not been involved in the salvage).

Please refer to appendix 4 for salvage notes on different materials. The

materials relating to this room / area can be copied from that document and

pasted into this section

23

Mould

Mould can grow in less than 48 hours, which is why objects should be air-dried

or frozen (if suitable) within this time.

It is not possible to identify a mould and whether it is harmful without

laboratory testing, so it is safer to treat all mould as if it were harmful. The

effects of breathing in mould spores over time are cumulative, and can lead to

health problems in later life. Always take precautions when mould is present.

• Wear nitrile / latex gloves when handling objects.

• Wear a face-mask that is suitable for use with mould spores and

particulates.

Stopping mould growth:

• Reduce humidity: Mould can form at 65% relative humidity (RH) if

there is poor ventilation. At 70-75% RH and above, mould will grow and

remain active. Use cold air fans to increase ventilation.

• Do not turn up the heat: This will make it grow faster.

• Dry or freeze wet collections: This will not kill the mould but it will

stop it growing until it can undergo conservation (not all objects can be

frozen; check the materials list in this plan or ask the advice of a

conservator).

• Do not use bleach or domestic products: These will cause

additional damage and will not keep the mould from recurring.

7. (Insert name of room or area here) Emergency plan

7.2 Flood (water leak from above)

Basic procedures

On discovering a flood/water leak, speak to (insert name of person who is

responsible for the building maintenance), who will isolate water mains or stop

the ingress of water.

Water leak (from above):

If water is dripping onto display cases:

• Assess the severity of the leak

And do either of the following:

• Protect (cover) the case with a large sheet of polythene and ensure

that the water does not pool near the case base

• Remove the object(s) from the case but you must have all the

equipment needed to hand before removing the case lid / opening the

door of the case.

If objects need to be moved:

• Are some objects more at risk than others? (i.e. on a lower shelf or

nearer the source of the leak)

• Are any of the objects at risk/affected on the Priority for salvage floor

plan?

Where are you going to move objects to? (treatment areas to assess

their condition):

Either of the following:

• (insert the name of the area or room) (onto floor or a table, which

should be padded with blankets or foam, covered with acid-free tissue

if objects are dry OR blankets and polythene if objects are damp/wet).

Cordon off area, so people coming into room are kept away from

objects.

• (insert the name of an alternative area or room) (onto floor or a table,

which should be padded with blankets or foam, covered with acid-free

tissue if objects are dry OR blankets and polythene if objects are

damp/wet). Cordon off area, so people coming into room are kept away

from objects.

(Think about any local authority, other museum, friends or neighbour’s

buildings that you may be able to arrange to use in the event of a major

emergency where you need to move collections off site)

25

IMPORTANT: dry objects must be kept away from damp/wet objects. This

needs to be assessed as objects are taken to treatment areas. Ideally there

should be two areas: dry treatment area and damp/wet treatment area.

• Prepare treatment areas before objects are removed from cases.

What do you need to move the objects?:

• Security screw drivers/ display case keys Important: case lids to

be left on/ doors to remain closed until objects are ready to be

removed.

• Latex/ nitrile gloves to be worn when handling objects.

• Scalpel/ small scissors (to remove objects that are tied onto mounts).

Do all objects need to be removed from mounts or can they be moved

safely still attached to the mounts?

• Trolleys for moving boxes/ crates.

• Boxes/ crates for moving smaller objects. If there are not enough,

objects will have to be unpacked at the treatment area and the boxes

re-used.

(Adapt this section for the specific room or area)

26

Salvage notes

Air-drying:

• Can be undertaken with objects laid out over an area.

• Bread crates (stackable) can be used to lay objects in, as they allow

air to circulate.

• A wind-tunnel can be created to aid drying but is not for use with

objects that require slow air-drying. For example, polythene sheets

can be draped over tables and the objects laid out beneath the table. A

fan can be placed underneath the table at one end but the air must not

directly blow onto any objects. Objects that require quicker drying can

be placed nearer the fan.

Dehumidifiers:

• Can be used in rooms to help dry out rooms that have been damaged

by flooding (or are water damaged after a fire).

• If used, the doors and windows of that room must be closed.

• The dehumidifier(s) must be plumbed in/or be emptied regularly but

they must not collect the water in open containers, as the moisture will

just be recycled back into the room.

DO NOT ADD HEAT DUE TO INCREASED RISK OF MOULD GROWTH

SEE NEXT SECTION FOR: SALVAGE NOTES (materials)

27

Salvage notes

• If objects are dry: keep them away from any damp/ wet objects

• If objects are damp/ wet: contact your Museums development Officer

who can provide you with the details of a Conservator, who will be able

to advise you (if he/she has not been involved in the salvage)

Please refer to appendix 4 for salvage notes on different materials. The

materials relating to the each room / area can be copied from that document

and paste into this section

28

Mould

Mould can grow in less than 48 hours, which is why objects should be airdried

or frozen (if suitable) within this time.

It is not possible to identify a mould and whether it is harmful without

laboratory testing, so it is safer to treat all mould as if it were harmful. The

effects of breathing in mould spores over time are cumulative, and can lead to

health problems in later life. Always take precautions when mould is present.

• Wear nitrile / latex gloves when handling objects.

• Wear a face-mask that is suitable for use with mould spores and

particulates.

Stopping mould growth:

• Reduce humidity: Mould can form at 65% relative humidity (RH) if

there is poor ventilation. At 70-75% RH and above, mould will grow and

remain active. Use cold air fans to increase ventilation.

• Do not turn up the heat: This will make it grow faster.

• Dry or freeze wet collections: This will not kill the mould but it will

stop it growing until it can undergo conservation (not all objects can be

frozen; check the materials list in this plan or ask the advice of a

conservator).

• Do not use bleach or domestic products: These will cause

additional damage and will not keep the mould from recurring.

29

7. (Insert name of room or area here) Emergency plan

7.3 Flood (Rising water)

Basic procedures

On discovering a flood, speak to (insert to person who is responsible for the

building maintenance), who will isolate water mains or stop the ingress of

water.

Rising water:

• Assess the severity of the flood:

• How deep is the water?

• Is the water level still rising? If so, how fast?

(Add a few sentences here about the potential risk of severe flooding, whether

it has been assessed as being serious etc. Also, whether it is likely that the

flood waters will reach a height as to cause damage to objects.)

If objects need to be moved:

• Are some objects more at risk than others? (i.e. on a lower shelf or

nearer the source of the flood)

• Are any of the objects at risk/affected on the Priority for salvage floor

plan?

Where are you going to move objects to (treatment areas to assess their

condition)?:

Either of the following (if unaffected/likely to be unaffected by flooding):

• (insert the name of the area or room) (onto floor or a table, which

should be padded with blankets or foam, covered with acid-free tissue

if objects are dry or blankets and polythene if objects are damp/wet).

Cordon off area, so people coming into room are kept away from

objects.

• (insert the name of an alternative area or room) (onto floor or a table,

which should be padded with blankets or foam, covered with acid-free

tissue if objects are dry or blankets and polythene if objects are

damp/wet). Cordon off area, so people coming into room are kept away

from objects.

• Prepare treatment areas before objects are removed from cases

(Think about any local authority, other museum, friends or neighbour’s

buildings that you may be able to arrange to use in the event of a major

emergency where you need to move collections off site)

30

IMPORTANT: dry objects must be kept away from damp/wet objects. This

needs to be assessed as objects are taken to treatment areas. Ideally there

should be two areas: dry treatment area and damp/wet treatment area.

What do you need to move the objects?:

• Wellington boots (in correct size, otherwise tripping when handling

objects is a risk).

• Security screw drivers/ display case keys Important: case lids to

be left on / doors to remain closed until objects are ready to be

removed.

• Latex/ nitrile gloves to be worn when handling objects.

• Scalpel/ small scissors (to remove objects that are tied onto mounts).

Do all objects need to be removed from mounts or can they be moved

safely still attached to the mounts?

• Boxes/ crates for moving smaller objects. If there are not enough,

objects will have to be unpacked at the treatment area and the boxes

re-used.

• Trolleys for moving grey bins/boxes (to be used in dry areas only).

(Adapt this section for the specific room or area)

If the flood in this area is serious:

• The person in charge must check Health and Safety procedures, and

undertake risk assessments.

• The area may have to be pumped out by the Fire Brigade and salvage

may not be able to start until the area is deemed safe. If this is the

case, use the time to prepare salvage/treatment areas.

• Form a human-chain (if enough people are available) to move

crates/objects. This saves person time and prevents congestion in tight

areas.

31

Salvage notes

Air-drying:

• Can be undertaken with objects laid out over an area.

• Bread crates (stackable) can be used to lay objects in, as they allow

air to circulate.

• A wind-tunnel can be created to aid drying but is not for use with

objects that require slow air-drying. For example, polythene sheets

can be draped over tables and the objects laid out beneath the table. A

fan can be placed underneath the table at one end but the air must not

directly blow onto any objects. Objects that require quicker drying can

be placed nearer the fan.

Dehumidifiers:

• Can be used in rooms to help dry out rooms that have been damaged

by flooding (or are water damaged after a fire).

• If used, the doors and windows of that room must be closed.

• The dehumidifier(s) must be plumbed in/or be emptied regularly but

they must not collect the water in open containers, as the moisture will

just be recycled back into the room.

DO NOT ADD HEAT DUE TO INCREASED RISK OF MOULD GROWTH

32

Salvage notes

• If objects are dry: keep them away from any damp/ wet objects

• If objects are damp/ wet: contact your Museums development Officer

who can provide you with the details of a Conservator, who will be able

to advise you (if he/she has not been involved in the salvage)

Please refer to appendix 4 for salvage notes on different materials. The

materials relating to the each room / area can be copied from that document

and paste into this section

33

Mould

Mould can grow in less than 48 hours, which is why objects should be airdried

or frozen (if suitable) within this time.

It is not possible to identify a mould and whether it is harmful without

laboratory testing, so it is safer to treat all mould as if it were harmful. The

effects of breathing in mould spores over time are cumulative, and can lead to

health problems in later life. Always take precautions when mould is present.

• Wear nitrile / latex gloves when handling objects.

• Wear a face-mask that is suitable for use with mould spores and

particulates.

Stopping mould growth:

• Reduce humidity: Mould can form at 65% relative humidity (RH) if

there is poor ventilation. At 70-75% RH and above, mould will grow and

remain active. Use cold air fans to increase ventilation.

• Do not turn up the heat: This will make it grow faster.

• Dry or freeze wet collections: This will not kill the mould but it will

stop it growing until it can undergo conservation (not all objects can be

frozen; check the materials list in this plan or ask the advice of a

conservator).

• Do not use bleach or domestic products: These will cause

additional damage and will not keep the mould from recurring.

34

7.4 Priority for salvage floor plan

The floor plan for this room / area, showing pictures and descriptions of

priority objects should be inserted here. See appendices 6 and 7 for guidance.

35

7.5 Salvage recovery sheet (headings only)

| Room name / no. (salvage location) | Object name / artist Object no. Priority object? | Object no. | Priority object? (y/n) | On loan to museum? (y/n) |
| --- | --- | --- | --- | --- |
| Treatment location? (wet/damp or dry) | Needs specialist conservation treatment (e.g. freezing)? (y/n) |

Sections 8/9…

Repeat step 7 for all rooms/site areas

10. Emergency Plan: Insect and mould

infestations

10.1 Insect infestation (actual or suspected)

Basic procedures

On discovering a suspected or actual insect infestation in your collections,

follow these procedures. Isolate the affected object(s) from the rest of the

collection; if the object is on loan to your institution or does not belong to your

museum, contact the lender or owner before any further treatment is

undertaken.

Suspected insect infestation

These guidelines are to be used if evidence of insects has been found on or

around the object, such as moth cases or woolly bear skins, and / or there is

damage to the object itself, but it is uncertain if it is fresh.

Isolate the object(s), suspected as having a problem, from others around

them. This will:

• Prevent any potential infestation from spreading

• Allow you to monitor the object to see if an infestation is present

To isolate objects:

• Smaller objects:

1. Place individual objects inside a polyethylene bag, squeeze out

as much air as possible and seal up the opening of the bag with

parcel tape.

2. Label the bag with the object details (name and object number),

the date of when it was placed in the bag and a sign saying ‘Do

not open- monitoring for insect infestation’.

• Larger objects (too large to be placed in a bag):

1. Place each object on a large sheet of polyethylene and ‘wrap’

the object, so that it is completely enclosed and seal with parcel

tape. Ensure that the edges of the polyethylene are folded over

several times before they are securely taped. Try to remove as

much air as possible from the package.

2. Label the package with the object details (name and object

number), the date of when it was placed in the package and a

sign saying ‘Do not open- monitoring for insect infestation’.

Monitor the object. If there are signs of an active infestation contact your

Museums Development Officer, who will be able to advise you on treating the

object.

39

Actual insect infestation

These guidelines are to be used if insects have been found on the object or

fresh damage is seen.

Isolate the object(s) from others around them. This will:

• Prevent the infestation from spreading

To isolate objects:

• Smaller objects:

1. Place individual objects inside a polyethylene bag, squeeze out

as much air as possible and seal up the opening of the bag with

parcel tape.

2. Label the bag with the object details (name and object number),

the date of when it was placed in the bag and a sign saying ‘Do

not open- insect infestation’.

• Larger objects (too large to be placed in a bag):

1. Place each object on a large sheet of polyethylene and ‘wrap’

the object, so that it is completely enclosed and sealed with

parcel tape. Ensure that the edges of the polyethylene are

folded over several times before they are securely taped. Try to

remove as much air as possible from the package.

2. Label the package with the object details (name and object

number), the date of when it was placed in the package and a

sign saying ‘Do not open- insect infestation’.

Contact your Museums Development Officer, who will be able to advise you

on treating the object.

40

10.2 Mould Infestation

Basic procedures

On discovering a mould infestation in your collections, follow these

procedures. Isolate the affected object(s) from the rest of the collection; if the

object is on loan to your institution or does not belong to your museum,

contact the lender or owner before any further treatment is undertaken.

Checking for mould

• Wear nitrile / latex gloves when handling objects. Wear a face-mask

that is suitable for use with mould spores and particulates.

• Check if the material feels damp and/or there is a mouldy smell.

• In the early stages of growth, mould appears as a fine web of

filaments (hyphae), often easily confused with dust, dirt and cobwebs.

This is a general rule, but there are exceptions.

• In the later stages, mould develops a bushy appearance, and fruiting

bodies containing spores can be seen under magnification. This is a

general rule, but there are exceptions.

Testing for mould

• Brush the mould with a fine natural hair brush (not a brush with

synthetic bristles) to see if the mould is dormant (dry and powdery)

or active (soft and smeary). Active mould will continue to grow and

damage collections. This is a general rule, but there are exceptions.

• Dormant mould will cause no further damage unless relative humidity

increases to 70-75% or more, when spores germinate and mould

becomes active. This is a general rule, but there are exceptions.

• Foxing on paper can be confused with mould. Foxing involves various

agents of bio-deterioration, including mould. It appears as red-brown

stains in discrete spots or irregular splotches, usually with no visible

hyphae or mould structure. It appears in susceptible papers exposed to

high relative humidity.

Mould infestation

These guidelines are to be used if evidence of mould has been found on or

around the object.

Isolate the object(s) from others around them. This will:

• Prevent any infestation from spreading

41

To isolate objects:

• Smaller objects:

1. Wear nitrile/ latex gloves when handling objects with a mould

infestation. Also wear a face-mask, which is suitable for mould

spores and particulates.

2. Place individual objects inside a polyethylene bag, squeeze out

as much air as possible and seal up the opening of the bag with

parcel tape.

3. Label the bag with the object details (name and object number),

the date of when it was placed in the bag and a sign saying ‘Do

not open- mould infestation’.

4. Place somewhere cool. The object should not remain in the bag

for a long time as this will encourage further mould growth and

damage, so contact your Museum Development Officer or

conservator as soon as possible.

• Larger objects (too large to be placed in a bag):

1. Wear nitrile/ latex gloves when handling objects with a mould

infestation. Also wear a face-mask, which is suitable for mould

spores and particulates.

2. Place each object on a large sheet of polyethylene and ‘wrap’

the object, so that it is completely enclosed and sealed with

parcel tape. Ensure that the edges of the polyethylene are

folded over several times before they are securely taped. Try to

remove as much air as possible from the package.

3. Label the package with the object details (name and object

number), the date of when it was placed in the package and a

sign saying ‘Do not open- mould infestation’.

4. Place somewhere cool. The object should not remain in the bag

for a long time as this will encourage further mould growth and

damage, so contact your Museum Development Officer or

conservator as soon as possible.

Stopping mould growth

• Reduce humidity: Mould can form at 65% relative humidity (RH) if

there is poor ventilation. At 70-75% RH and above, mould will grow and

remain active.

• Do not turn up the heat. This will make it grow faster.

• Dry or freeze wet collections. This will not kill the mould but it will

stop it growing until it can undergo conservation (not all objects can be

frozen; check the materials list in this plan or ask the advice of a

conservator).

• Do not use bleach or domestic products. These will cause additional

damage and will not keep the mould from recurring.

11. Emergency Plan: Theft and vandalism

11.1 Theft

Theft from museum collection

If theft is currently occurring: dial 999

When you suspect an object has been stolen:

• Report the loss to the person in charge of security at your museum;

they should then report this to the head of the museum.

• The head of the museum should contact the staff from different

departments / sections of the museum, to ensure that the object has

not been removed from its usual position for another reason (such as

conservation treatment etc).

If it is determined that the object has been stolen:

• The head of the museum must call the local Police (insert telephone

number here) to report the incident.

• Do not move anything from the area until the police have been to the

museum.

• Telephone your insurers to inform them of the loss.

Insurance company: (Insert details here)

Contact name: (Insert details here)

Telephone number: (Insert details here)

• If the object is on loan to your museum or does not belong to the

museum, contact the lenders to tell them of the situation.

To help with enquiries:

• Provide any photographs of the object to aid identification; also provide

dimensions and any other information that you think is relevant.

• Provide the Police with CCTV footage, if possible.

If the building has been damaged by the thief:

• Make the building secure temporarily, until proper repairs can be

undertaken.

43

11.2 Vandalism

Vandalism of museum building and collection

If vandalism is currently occurring: dial 999

Building:

When you notice that vandalism has occurred:

• Report the damage to the person in charge of security at your museum

• Call the local Police (insert telephone number here) to report the

incident.

• If relevant, telephone your insurers to inform them of the damage.

Insurance company: (Insert details here)

Contact name: (Insert details here)

Telephone number: (Insert details here)

To help with enquiries:

• Provide the Police with CCTV footage, if possible.

If the building has been damaged by the vandal(s):

• Make the building secure temporarily, until proper repairs can be

undertaken.

Collections

If vandalism is currently occurring: dial 999

When you notice that vandalism has occurred:

• Report the damage to the person in charge of security at your museum,

who should then contact the head of the museum

• The head of the museum should call the local Police (insert telephone

number here) to report the incident

• Telephone your insurers to inform them of the damage

To help with enquiries:

• Provide the Police with CCTV footage, if possible

• Contact a conservator and arrange for them to assess the condition of

the object.

• Do not move the object until the Police have been to the museum

44

12. Emergency Plan: Security threats

12.1 Security threats to your museum

These security threats could include the following:

• A threatening call directly to the building

• A threatening call received elsewhere and notified to you by the police

• The discovery of a suspicious item in the building

• The discovery of a suspicious item or vehicle outside your building

• The discovery of a suspicious item elsewhere notified to you by

neighbours or the Police

The decision to evacuate your building will usually be undertaken by the

museum concerned, but the Police will also advise you.

The evacuation procedures for this situation will differ to those required

for a fire:

• Your assembly point needs to be much further away than that for a fire,

at least 500 metres from the building (but car parks should not be

used).

• Your evacuation route will have to be flexible, depending on where the

threat is.

You can minimise the risks of these threats by undertaking the

following:

• Consider displaying notices for staff and visitors, reminding them not to

leave bags or personal belongings unattended.

• Think about anything in your buildings or collections that may cause

offence or be controversial to some people.

45

13. Suppliers & contacts

(Insert name, address, telephone and fax numbers of the following types of

companies. These are only suggestions – please insert any information that is

relevant to your museum)

Building Maintenance / Repairs

• Builders

• Plumbers

• Electricians

Builders Merchants

Conservation Organisations

• Drying equipment

• Freeze-drying companies

Conservators

• For specific materials relevant to your collections

• Local and specialists further afield

Conservation & Salvage Supplies

• Crate Hire

• Dehumidifier suppliers

• Demolition Companies

Drain Cleaning

Essential Services

• Electricity company

• Gas company

• Water company

• Telephone company

• Security company / intruder alarms

46

Fans

Glass Replacement Companies

Industrial Cleaning Equipment

Insurance Company

Locksmiths

Portable Radio Hire Companies

Pump Hire

Removal Companies

Safety / Protective Clothing

Skip Hire

Temporary Accommodation / Storage

• Local schools, town or village halls, sports centres

• Marquee hire

• Commercial estate / letting agents

Tool Hire

Vacuum Cleaners (Wet and Dry)

24-hour Plant Hire

47

14. Salvage kit

A salvage kit is a selection of equipment and materials that are stored

(preferably not in your main museum building) for use in an emergency

(however small or large). The contents should reflect the greatest risks

identified by the risk assessment exercise. It should be easily accessible 24

hours a day, though kept in a secure area so that equipment cannot be taken

from it and not replaced.

Location of salvage kit

Location name: (Insert details here)

Location address: (Insert details here)

Room number: (Insert details here)

Key number / name: (Insert details here)

Key kept where?: (Insert details here)

(Add any further information that is relevant)

Person responsible for upkeep of kit

Name: (Insert details here)

Daytime tel: (Insert details here)

Home tel: (Insert details here)

Mobile: (Insert details here)

Date last checked

Date:

Items needing replacing?:

Objects replaced when?:

Date next due

Date: 1 year from above date

Appendices

Risk assessment

Building maintenance checklist

Emergency response team: roles

Salvage notes: Materials

Policy: selection of priority objects for salvage

Floor plans showing priority objects

Salvage kit contents